# Introducing the USB software interface

Put

EP1

Get

EP1

InitUSB

USB Peripheral

Main Application

USB

Microchip provides a layer of software for the PIC16C745/65 that handles the lowest level interface so your application won’t have to.  This provides a simple Put/Get interface for communication.  Most of the USB processing takes place in the background through the Interrupt Service Routine.  From the application viewpoint, the enumeration process and data communication takes place without further interaction.  However substantial setup is required in the form of generating appropriate descriptors.

# Integrating USB into your application

The latest version of the USB interface software is available on Microchip’s website. See http://www.microchip.com/

The interface to the application is packaged in 5 functions: InitUSB, PutEP1, PutEP2, GetEP1 and GetEP2.  InitUSB initializes the USB peripheral allowing the host to enumerate the device.  Then for normal data communications, the PutEPn functions send data to the host and the GetEPn functions receive data from the host.

There’s also a fair amount of setup work that must be completed.  USB depends heavily on the descriptors.  These are the software parameters that are communicated to the host to let it know what the device is, and how to communicate with it.  See USB V1.1 spec section 9.5 for more details.

InitUSB enables the USB interrupt so enumeration can begin.  The actual enumeration process occurs in the background, driven by the host and interrupt service routine.  Macro ConfiguredUSB waits until the device is in the CONFIGURED state.  The time required to enumerate is completely dependent on the host and bus loading.

# Interrupt structure concerns

## Processor Resources

Most of the USB processing occurs via the interrupt and thus is invisible to the application.  However it still consumes processor resources.  These include ROM, RAM, Common RAM, Stack Levels and processor cycles.  This section attempts to quantify the impact on each of these resources, and shows ways to avoid conflicts.

These considerations should be taken into account if you write your own Interrupt Service Routine: Save W, STATUS, FSR and PCLATH which are the file registers that may be corrupted by servicing the USB interrupt.

The file usb_main.c provides a skeleton ISR which does this for you, and includes tests for each of the possible ISR bits.  This provides a good starting point if you haven’t already written your own.

## Stack Levels

The hardware stack on the PIC is only 8 levels deep.  So the worst case call between the application and ISR can only be 8 levels.  The enumeration process requires 4 levels, so it’s best if the main application holds off on any processing until enumeration is complete.  ConfiguredUSB is a macro that waits until the enumeration process is complete for exactly this purpose.  This macro does this by testing the lower two bits of USWSTAT (0x197).

## ROM

The code required to support the USB interrupt, including the chapter 9 interface calls, but not including the HID interface calls or descriptor tables is about 2kW.  The HID specific functions and descriptor tables take up additional memory.  The location of these parts is not restricted, and the linker script may be edited to control the placement of each part.  See the Strings and Descriptors sections in the linker script.

## RAM

With the exception of Common RAM discussed below, servicing the USB interrupt requires approximately 39 bytes of  RAM in Bank 2.  That leaves all the General Purpose RAM in banks zero and one, plus half of Bank 2 available for your application to use.

## Common RAM usage

The PIC16C745/765 has 16 bytes of common RAM.  These are the last 16 addresses in each bank and all refer to the same 16 bytes of memory without regard to which register bank is currently addressed by the RP0, RP1 and IRP bits.

These are particularly useful when responding to interrupts.  When an interrupt occurs, the ISR doesn’t immediately know which bank is addressed.  With devices that don’t support common RAM, the W register must be provided for in each bank.  The PIC16C745/765 can save the appropriate registers in Common RAM and not have to waste a byte in each bank for the W register.

## Buffer allocation

The PIC16C745/765 has 64 bytes of Dual Port RAM.  24 are used for the Buffer Descriptor Table, (BDT) leaving 40 bytes for buffers.

Endpoint 0 (EP0) IN and OUT need dedicated buffers since a setup transaction can never be NAKed.  That leaves three buffers for four possible endpoints. But the USB spec requires that low speed devices are only allowed 2 endpoints (USB V1.1 paragraph 5.3.1.2), where an endpoint is a simplex connection that is defined by the combination of endpoint number and direction.

The default configuration allocates individual buffers to EP0 OUT, EP0 IN, EP1 OUT, and EP1 IN.  The last buffer is shared between EP2 IN and EP2 OUT.  Again, the spec says low speed devices can only use 2 endpoints beyond EP0.  This configuration supports most of the possible combinations of endpoints (EP1 OUT and EP1 IN, EP1 OUT and EP2 IN, EP1 OUT and EP2 OUT, EP1 IN and EP2 OUT, EP1 IN and EP2 IN).  The only combination that is not supported by this configuration is EP2 IN and EP2 OUT.  If your application needs both EP2 IN and EP2 OUT, the function USBReset will need to be edited to give each of these endpoints dedicated buffers at the expense of EP1.

# File packaging

The software interface is packaged into five files, designed to simplify the integration with your application.  These are:

- usb_main.c

- usb_ch9.c

- usb.h

- usb_defs.h

File usb_main.c is useful as a starting point on a new application and as an example of how an existing application needs to service the USB interrupt and communicate with the core functions.

File usb_ch9.c contains the interface and core functions needed to enumerate the bus.  It also contains the functions that service the USB, send data to the host, and receive data from the host.

# Function Call reference

Interface between the Application and Protocol layer takes place in five main functions: InitUSB, PutEP1, PutEP2, GetEP1 and GetEP2.

InitUSB should be called by the main program after the device is powered up.  Be sure to precede the call with a 16 μs delay to give the SIE a chance to reset before beginning enumeration.  InitUSB enables the USB peripheral and the USB reset interrupt.  It also transitions the part to the powered state in order to prepare the device for enumeration.  Before enumeration is initiated, the USB Reset is the only USB interrupt allowed, preventing the part from responding to anything on the bus until it’s been reset.  The USB Reset interrupt initializes the Buffer Descriptor Table (BDT) and transitions the part to the default state where it responds to commands on address zero.  When it receives a SET ADDRESS command, the device transitions to the addressed state and now responds to commands on the new address.

PutEPn(argument1, argument2) sends data to the host.  Call this function with the following specified:

- The number of bytes to send in argument2.

- Buffer pointer in argument1.

If the IN buffer is available for that endpoint, the block of data is copied to the buffer, then the Data 0/1 bit is flipped and the OWNs bit is set.  Should the buffer not be available, a zero will be returned so the application can try again later.  If the function is successful it will return a one.

GetEPn(argument) returns data sent from the host.  Call this function with the buffer pointer in the argument.  If there is a buffer ready, meaning data has been received from the host, it is copied to the destination pointed to by the argument.  If no data is available, this function returns a zero.  If the function is successful it will return a one.

ServiceUSBInt handles all interrupts generated by the USB peripheral.  First it copies the active buffer to common RAM which provides a quick turn around on the buffer in dual port RAM and also to avoids having to switch banks during processing of the buffer.

StallUSBEP/UnstallUSBEP sets or clears the stall bit in the endpoint control register.  The stall bit indicates to the host that user intervention is required and until such intervention is made, further attempts to communicate with the endpoint will not be successful.  Once the user intervention has been made, UnstallUSBEP will clear the bit allowing communications to take place.  These calls are used to signal to the host that user intervention is required.  An example of this might be a printer out of paper.

SoftDetachUSB clears the DEV_ATT bit, electrically disconnecting the device from the bus, then reconnecting so it can be re-enumerated by the host.  This process takes approximately 50 mS, to ensure that the host has seen the device disconnect and reattach to the bus.

The ConfiguredUSB macro continuously polls the enumeration status bits and waits until the device has been configured by the host.  This should be used after the call to InitUSB and prior to the first time your application attempts to communicate on the bus.

# Behind the scenes:

InitUSB clears the error counters and enables the 3.3 V regulator and the USB reset interrupt.  This implements the requirement to prevent the PIC from responding to commands until the device has been reset.

The host sees the device, and resets the device to begin the enumeration process.  The reset then initializes the Buffer Descriptor Table (BDT) and Endpoint Control Registers in addition to enabling the remaining USB interrupt sources.

The Interrupt transfers control to the interrupt vector (address  0x0004).  Any interrupt service routine must preserve the processor state by saving the FSRs that might change during interrupt processing.  We recommend saving W, STATUS, PCLATH and FSR.  W can be stored in Common RAM to avoid banking issues.  Then it starts polling the Interrupt flags to see what triggered the interrupt.  The USB interrupts are serviced by calling ServiceUSBInt which further tests the USB interrupt sources to determine how to process the interrupt.

Then the host sends a setup token requesting the device descriptor.  The USB Peripheral receives the Setup transaction, places the data portion in the EP0 out buffer, loads the USTAT register to indicate which endpoint received the data and triggers the Token Done (TOK_DNE) interrupt.  The Chapter 9 commands then interpret the Setup token and sets up the data to respond to the request in the EP0 IN buffer.  It then sets the UOWN bit to tell the SIE there is data available.

Next, the host sends an IN transaction to receive the data from the setup transaction.  The SIE sends the data from the EP0 IN buffer and then sets the Token done interrupt to notify us that the data has been sent.  If there is additional data, the next buffer full is set up in the EP0 IN buffer.

This token processing sequence holds true for the entire enumeration sequence, which walks through the flow chart starting chapter 9 of the USB spec.  The device starts off in the powered state, transitions to RESET via the reset interrupt, transitions to ADDRESSED via the Set Address command, and transitions to CONFIGURED via a Set Configuration command.

The USB peripheral detects several different errors and handles most internally.  The USB_ERR interrupt notifies the PIC that an error has occurred.  No action is required by the PIC when an error occurs.  Instead the errors are simply acknowledged and counted.  There is no mechanism to pull the device off the bus if there are too many errors.  If this behavior is desired it must be implemented in the application.

The Activity interrupt is left disabled until the USB peripheral detects no bus activity for 3 mS.  Then it suspends the USB peripheral and enables the activity interrupt.  The activity interrupt then reactivates the USB peripheral when bus activity resumes so processing may continue.

# Examples

This example shows how the USB functions are used.  This example first initializes the USB peripheral which allows the host to enumerate the device.  The enumeration process occurs in the background, via an Interrupt service routine. ConfiguredUSB waits until enumeration is complete, and then polls EP1 OUT to see if there is any data available.  When a buffer is available, it is copied to the IN buffer.  Presumably your application would do something more interesting with the data than this example.

; ******************************************************************

; Demo program that initializes the USB peripheral, allows the Host

; 	to Enumerate, then copies buffers from EP1OUT to EP1IN.

; ******************************************************************

#define TRUE 1

void main() {

int buffer[8];

InitUSB();			//Set up everything so we can enumerate

ConfiguredUSB();		//wait here until we have enumerated.

while (TRUE) {

if (GetEP1(&buffer)) {

while (!PutEP1(&buffer, 0x08));

}

}

}

# Cursor Demonstration

Microchip provides a working USB demonstration.  This demonstration has the effect of moving the cursor in a small circle on the user’s screen.  The following steps will get the demonstration working with an actual part or the MPLAB ICE:

Getting the USB demonstration to work on a PIC16C745/65

- Unzip usbxxxnc.zip to a project folder.

- Build the project in MPLAB.

- Program a PIC16C745/765 using a PICSTART Plus or a PRO MATE II.  Make sure the configuration bits are set as follows:

- Oscillator: 		H4

- Watchdog Timer:	Off

- Power Up Timer:	Off

- Code Protect:	Off

- The figure at the bottom of this section details the circuit in which to implement the PIC.   Attach the USB cable to your computer.

- Provide power to the PIC.  If you are running Windows 98(with the USB upgrade), Windows NT,  Windows 2000, Windows XP, Mac OS9, or Mac OS10 then your operating system will detect a new device and install the necessary drivers automatically.  After this occurs you should see the cursor rotating in a small circle on your screen.  To stop the cursor from rotating, detach the USB cable.

Getting the USB demonstration to work using the MPLAB ICE

- Unzip usbxxxnc.zip to a project folder.

- Build the project in MPLAB.

- Make sure the emulator is set up as follows in the Development Mode dialog:

- Tools:				MPLAB-ICE Emulator

- Clock: Desired Frequency:		24 MHz

- Configuration: Watch Dog Timer:	None

- Power: Processor Power:		From Target Board

- The figure below shows the circuit in which to implement the ICE.  Connect the USB cable to your computer.  (The external oscillator portion of the circuit is optional when using MPLAB ICE.)

- Run the project on the emulator.  If you are running Windows 98(with the USB upgrade), Windows NT, Windows 2000, Windows XP,  Mac OS9, or Mac OS10 then your operating system will detect a new device and install the necessary drivers automatically.  After this occurs you should see the cursor rotating in a small circle on your screen.  To stop the cursor from rotating, press F5.

### Emulation Tips

- Turn on the emulator, then open the project to initialize the emulator.  If the project is in a mode other than Emulation follow step 2.

- If your project is already open before you turn the emulator on you can initialize it by first, turning the emulator on.  Then go to the option menu and click Development Mode.  Make sure that the MPLAB-ICE Emulator radio button is selected in the Tools folder of the Development Mode dialog.  Click OK.  The emulator will initialize.

- Make sure the proper processor module and device adapter are installed.

- Should you have other problems please refer to the MPLAB-ICE Users Guide as your first resource.

Note:  Be sure to pull D- up to VUSB (via R1) not VDD.  For more on why this is done see the section Universal Serial Bus:Transceiver:Regulator in the PIC16C745/65 datasheet.